LIGO Laboratory / LIGO Scientific Collaboration

LIGO-E1200784-v2	Advanced LIGO	4/3/2015

TwinCAT Library 
for the Advanced LIGO Timing System

Maggie Tse, Daniel Sigg

Distribution of this document:

LIGO Scientific Collaboration

This is an internal working note
of the LIGO Laboratory.

| California Institute of Technology LIGO Project – MS 18-34 1200 E. California Blvd. Pasadena, CA 91125 Phone (626) 395-2129 Fax (626) 304-9834 E-mail: info@ligo.caltech.edu | Massachusetts Institute of Technology LIGO Project – NW22-295 185 Albany St Cambridge, MA 02139 Phone (617) 253-4824 Fax (617) 253-7014 E-mail: info@ligo.mit.edu |
| --- | --- |
| LIGO Hanford Observatory P.O. Box 159 Richland WA 99352 Phone 509-372-8106 Fax 509-372-8137 | LIGO Livingston Observatory P.O. Box 940 Livingston, LA 70754 Phone 225-686-3100 Fax 225-686-7189 |

http://www.ligo.caltech.edu/

| Library |  |
| --- | --- |
| Title | TimingMasterFanout |
| Version | 1 |
| TwinCAT version | 2.11 |
| Name space | Timing |
| Author | Daniel Sigg |
| Description | The Advanced LIGO timing system consists of timing master synchronized to GPS with sixteen fanout ports, a set of timing fanout chassis and numerous timing slaves. The master and fanouts are the same same chassis configured differently. They contain serial RS422 ports which transmit the status information of the master-fanout chassis and each of its slave once every second. By reading the serial ports of all master-fanout chassis the health of the entire timing system can be monitored. The Advanced LIGO timing system has a wiki. It is described in E090003, E080541, E0900036, T070173 and T0900032. |
| Error Codes | As part of the TimingMasterFanoutStruct (master-fanout chassis): 0x0001 – Communication error 0x0002 – Data CRC 0x0004 – Data missing 0x0008 – Loss-of-signal 0x0010 – Uplink 0x0020 – Uplink CRC 0x0040 – VCXO control voltage 0x0080 – OCXO not locked 0x0100 – OCXO control voltage 0x0200 – GPS not locked 0x0400 – GPS time error 0x0800 – External PPS time eror 0x1000 – Uplink time error 0x2000 – Firmware error (not in svn or wrong revision) 0xnnnn0000 – PORT_i (bit encoded slave or port error) As part of the TimingFanoutStruct (port and slave module): 0x0001 – Downlink loss-of-signal 0x0002 – Downlink down 0x0004 – Downlink CRC error 0x0008 – Delay error 0x0010 – Missing delay 0x0020 – Ignored slave 0x0040 – SLAVE (implicates a slave error) As part of the TimingSlaveStruct (slave module): 0x0001 – Invalid or no data 0x0002 – Uplink loss-of-signal 0x0004 – Uplink down 0x0008 – Uplink CRC 0x0010 – VCXO control voltage 0x0020 – GPS time 0x0040 – Unknown slave 0x0080 – Firmware error (not in svn or wrong revision) 0x0100 – FANOUT (Indicates an error in the fanout slave) 0x0200 – DUOTONE (Indicates an error in the DuoTone slave) 0x0400 – CFC (Indicates an error in the CFC slave) 0x0800 – XOLOCKING (Indicates an error in the XOLOCKING slave) 0x1000 – IRIGB (Indicates an error in the IRIGB slave) As part of the TimingSlaveFanoutStruct (through the port): Not used at this time As part of the TimingSlaveDuoToneStruct slave: Not used at this time As part of the TimingSlaveCFCStruct slave: Not used at this time As part of the TimingSlaveIrigBStruct slave: Not used at this time As part of the TimingSlaveXOLockingStruct slave: Not used at this time |
| Library dependencies | SaveRestore, Error, ComlibV2 |

| Software Input/Output Type TYPE TimingSerialPortBufferStruct : STRUCT RxBuffer: ComBuffer; TxBuffer: ComBuffer; COMportControlError: BOOL; COMportControlErrorID: ComError_t; END_STRUCT END_TYPE |  |
| --- | --- |
| Type name | TimingSerialPortBufferStruct |
| Description | This structure facilitates the communication between the serial ports and the MasterTimingFanoutFB function block. The TimingSerialPortControlFB function block is used to communicate with the hardware. |
| Definition | STRUCT |
| Element | Name: RxBuffer Type: ComBuffer Description: Receive buffer of the serial port |
| Element | Name: TxBuffer Type: ComBuffer Description: Transmit buffer of the serial port |
| Element | Name: COMportControlError Type: BOOL Description: Communication error flag of the serial port hardware |
| Element | Name: COMportControlErrorID Type: ComError_t Description: Communication error code |

| User Interface Type TYPE TimingMasterFanoutStruct : STRUCT Error: ErrorStruct; Name: STRING; ComErr: BOOL; ComErrCount: INT; CRCErr: BOOL; CRCErrCount: INT; ComMissing: BOOL; ComMissCount: INT; DownTime: UDINT; ComCount: UDINT; ComLength: UDINT; BoardId: STRING(10); BoardRev: INT; Serial: UDINT; CodeId: STRING(10); CodeRev: UDINT; GPS: UDINT; Addr: UDINT; StrAddr: STRING(24); DIP: WORD; IsMaster: BOOL; HasFanout: BOOL; FanoutPorts: INT; HasOCXO: BOOL; HasExtPPS: BOOL; HasGPS: BOOL; UseExt: BOOL; UseGPS: BOOL; UseUplink: BOOL; GPSLocked: BOOL; OCXOLocked: BOOL; UplinkUp: BOOL; UplinkLOS: BOOL; UplinkErrCount: INT; UplinkCRCErr: BOOL; UplinkCRCErrCount: INT; VCXOCtrl: LREAL; OCXOCtrl: LREAL; OCXOErr: LREAL; UplinkDelay: LREAL; ExtPPSDelay: LREAL; TimingTolerance: LREAL; GPSErr: BOOL; GPSErrCount: INT; GPSLatitude: LREAL; GPSLongitude: LREAL; GPSHeight: LREAL; GPS3DSpeed: LREAL; GPS2DSpeed: LREAL; GPSHeading: LREAL; GPSDOP: LREAL; GPSVisSatellites: INT; GPSTrackSatellites: INT; GPSStatus: WORD; GPSFix: STRING(25); GPSNarrowband: BOOL; GPSAntenna: BOOL; GPSSerial: STRING(10); FanoutUp: WORD; FanoutLOS: WORD; FanoutMissing: WORD; FanoutDelayErr: WORD; Port: ARRAY[0..15] OF TimingFanoutStruct; Visual: STRING; END_STRUCT END_TYPE |  |
| --- | --- |
| Type name | TimingMasterFanoutStruct |
| Description | This is the main user interface structure containing all releventa information of a master-fanout chassis and the slave modules attached to it. |
| Definition | STRUCT |
| Input Tag | Name: Error Type:ErrorStruct Description: Error handling |
| In/out Tag | Name: Name Type: STRING Description: Name/location of MFO unit |
| Input Tag | Name: ComErr Type: BOOL Description: Communication error flag |
| Input Tag | Name: ComErrCount Type: INT Description: Count of the communication errors |
| Input Tag | Name: CRCErr Type: BOOL Description: CRC error in data from MFO |
| Input Tag | Name: CRCErrCount Type: INT Description: CRC error counter |
| Input Tag | Name: ComMissing Type: BOOL Description: Communication missing |
| Input Tag | Name: ComMissCount Type: INT Description: Communication missing count |
| Input Tag | Name: DownTime Type: UDINT Description: Time since last valid communication |
| Input Tag | Name: ComCount Type: UDINT Description: Number of received communication buffers |
| Input Tag | Name: ComLength Type: UDINT Description: Number of received bytes(will overflow) |
| Input Tag | Name: BoardId Type: STRING(10) Description: Board type |
| Input Tag | Name: BoardRev Type: INT Description: Board revision number |
| Input Tag | Name: Serial Type: UDINT Description: Board serial number |
| Input Tag | Name: CodeId Type: STRING(10) Description: Code type |
| Input Tag | Name: CodeRev Type: UDINT Description: Code revision number |
| Input Tag | Name: GPS Type: UDINT Description: GPS time in sec |
| Input Tag | Name: Addr Type: UDINT Description: Board address |
| Input Tag | Name: StrAddr Type: STRING(24) Description: Board address as string |
| Input Tag | Name: DIP Type: WORD Description: DIP switch settings |
| Input Tag | Name: IsMaster Type: BOOL Description: This is a timing master |
| Input Tag | Name: HasFanout Type: BOOL Description: Fanout ports are available |
| Input Tag | Name: FanoutPorts Type: INT Description: Number of fanout ports |
| Input Tag | Name: HasOCXO Type: BOOL Description: Contains an OCXO |
| Input Tag | Name: HasExtPPS Type: BOOL Description: An external PPS is available |
| Input Tag | Name: HasGPS Type: BOOL Description: Has an internal GPS receiver |
| Input Tag | Name: UseExt Type: BOOL Description: Uses the external PPS for timing source |
| Input Tag | Name: UseGPS Type: BOOL Description: Uses the internal GPS for timing source |
| Input Tag | Name: UseUplink Type: BOOL Description: Uses the uplink for timing source |
| Input Tag | Name: GPSLocked Type: BOOL Description: GPS receiver is locked |
| Input Tag | Name: OCXOLocked Type: BOOL Description: OCXO is locked to timing source |
| Input Tag | Name: UplinkUp Type: BOOL Description: Uplink port is up and running |
| Input Tag | Name: UplinkLOS Type: BOOL Description: Loss of signal in uplink port |
| Input Tag | Name: UplinkErrCount Type: INT Description: Error counter for uplink port |
| Input Tag | Name: UplinkCRCErr Type: BOOL Description: CRC error in uplink data |
| Input Tag | Name: UplinkCRCErrCount Type: INT Description: CRC error counter for uplink data |
| Input Tag | Name: VCXOCtrl Type: LREAL Description: VCXO control voltage |
| Input Tag | Name: OCXOCtrl Type: LREAL Description: OCXO control voltage |
| Input Tag | Name: OCXOErr Type: LREAL Description: OCXO Error |
| Input Tag | Name: UplinkDelay Type: LREAL Description: Uplink delay in sec |
| Input Tag | Name: ExtPPSDelay Type: LREAL Description: Ext PPS delay in sec |
| Input Tag | Name: TimingTolerance Type: LREAL Description: Timing tolerance in sec |
| Input Tag | Name: GPSErr Type: BOOL Description: GPS error flag |
| Input Tag | Name: GPSErrCount Type: INT Description: Error counter for GPS errors |
| Input Tag | Name: GPSLatitude Type: LREAL Description: GPS latitude -90° to +90° |
| Input Tag | Name: GPSLongitude Type: LREAL Description: GPS longitude -180° to +180° |
| Input Tag | Name: GPSHeight Type: LREAL Description: GPS height -1000m to +18000m |
| Input Tag | Name: GPS3DSpeed Type: LREAL Description: GPS 3D speed 0m/s to 514m/s |
| Input Tag | Name: GPS2DSpeed Type: LREAL Description: GPS 2D speed 0m/s to 514m/s |
| Input Tag | Name: GPSHeading Type: LREAL Description: GPS heading 0° to 360° |
| Input Tag | Name: GPSDOP Type: LREAL Description: GPS dilution of precision 0 to 99.9 |
| Input Tag | Name: GPSVisSatellites Type: INT Description: Number of visible satellites |
| Input Tag | Name: GPSTrackSatellites Type: INT Description: Number of tracked satellites |
| Input Tag | Name: GPSStatus Type: WORD Description: GPS status |
| Input Tag | Name: GPSFix Type: STRING(25) Description: GPS fix message |
| Input Tag | Name: GPSNarrowband Type: BOOL Description: GPS in narrowband mode |
| Input Tag | Name: GPSAntenna Type: BOOL Description: GPS antenna OK |
| Input Tag | Name: GPSSerial Type: STRING(10) Description: GPS receiver serial number |
| Input Tag | Name: FanoutUp Type: WORD Description: Port up and running (bit encoded) |
| Input Tag | Name: FanoutLOS Type: WORD Description: Loss-of-signal in ports (bit encoded) |
| Input Tag | Name: FanoutMissing Type: WORD Description: Missing delay in ports (bit encoded) |
| Input Tag | Name: FanoutDelayErr Type: WORD Description: Delay error in port (bit encoded) |
| Mixed | Name: Port Type: ARRAY[0..15] OF TimingFanoutStruct Description: Array of port information (16 ports) |
| Input Tag | Name: Visual Type: STRING Description: Name of the uplink MFO visual |

| User Interface Subtype TYPE TimingFanoutStruct : STRUCT Error: ErrorStruct; Active: BOOL; DownTime: UDINT; MeasuredDelay: LREAL; MeasuredAdvance: LREAL; UsedAdvance: LREAL; Up: BOOL; LOS: BOOL; DelayErr: BOOL; Missing: BOOL; ErrCount: INT; CRCErr: BOOL; CRCErrCount: INT; Slave: TimingSlaveStruct; END_STRUCT END_TYPE |  |
| --- | --- |
| Type name | TimingFanoutStruct |
| Description | This structure describes a single port of the master-fanout chassis. It includes the both the port information itself as well as the slave information received through the port. |
| Definition | STRUCT |
| Input Tag | Name: Error Type: ErrorStruct Description: Error handler |
| In/Out Tag | Name: Active Type: BOOL Description: Port is actively used |
| Input Tag | Name: DownTime Type: UDINT Description: Port down time in sec |
| Input Tag | Name: MeasuredDelay Type: LREAL Description: Measured fiber round-trip delay in sec |
| Input Tag | Name: MeasuredAdvance Type: LREAL Description: Timing advance from measured delay |
| Input Tag | Name: UsedAdvance Type: LREAL Description: Used timing advance |
| Input Tag | Name: Up Type: BOOL Description: Port is up and running |
| Input Tag | Name: LOS Type: BOOL Description: Loss of signal in port |
| Input Tag | Name: DelayErr Type: BOOL Description: Delay error in port |
| Input Tag | Name: Missing Type: BOOL Description: Missing delay measurement in port |
| Input Tag | Name: ErrCount Type: INT Description: Error counter for port |
| Input Tag | Name: CRCErr Type: BOOL Description: CRC error in downlink data |
| Input Tag | Name: CRCErrCount Type: INT Description: CRC error counter for downlink data |
| In/Out Tag | Name: Slave Type: TimingSlaveStruct Description: Describes the slave information received by the port |

| User Interface Subtype TYPE TimingSlaveStruct : STRUCT Error: ErrorStruct; Name: STRING; IsFanout: BOOL; IsDuotone: BOOL; IsCFC: BOOL; IsXOLocking: BOOL; IsIRIGB: BOOL; BoardId: STRING(10); BoardRev: INT; Serial: UDINT; CodeId: STRING(10); CodeRev: UDINT; GPS: UDINT; Addr: UDINT; StrAddr: STRING(24); DIP: WORD; UplinkUp: BOOL; UplinkLOS: BOOL; UplinkErrCount: INT; UplinkCRCErr: BOOL; UplinkCRCErrCount: INT; VCXOCtrl: LREAL; Id: TimingSlaveEnum; Generic: TimingSlaveGenericStruct; Fanout: TimingSlaveFanoutStruct; DuoTone: TimingSlaveDuoToneStruct; CFC: TimingSlaveCfcStruct; XOLock: TimingSlaveXoLockingStruct; IRIGB: TimingSlaveIrigBStruct; END_STRUCT END_TYPE |  |
| --- | --- |
| Type name | TimingSlaveStruct |
| Description | This structure describes a slave module. It contains information about a particular slave which is identified by the Id tag. All slaves use fill out the generic structure as well as the one corresponding to their type. Inappropriate slave structures are filled with zeroes. |
| Definition | STRUCT |
| Input Tag | Name: Error Type: ErrorStruct Description: Error handling |
| In/Out Tag | Name: Name Type: STRING Description: Name/location of MFO unit |
| Input Tag | Name: IsFanout Type: BOOL Description: This is a fanout chassis |
| Input Tag | Name: IsDuotone Type: BOOL Description: This is a DuoTone |
| Input Tag | Name: IsCFC Type: BOOL Description: This is a comparator/frequency counter |
| Input Tag | Name: IsXOLocking Type: BOOL Description: This is an RF source |
| Input Tag | Name: IsIRIGB Type: BOOL Description: This is an IRIG-B |
| Input Tag | Name: BoardId Type: STRING(10) Description: Board type |
| Input Tag | Name: BoardRev Type: INT Description: Board revision number |
| Input Tag | Name: Serial Type: UDINT Description: Board serial number |
| Input Tag | Name: CodeId Type: STRING(10) Description: Code type |
| Input Tag | Name: CodeRev Type: UDINT Description: Code revision number |
| Input Tag | Name: GPS Type: UDINT Description: GPS time in sec |
| Input Tag | Name: Addr Type: UDINT Description: Board address |
| Input Tag | Name: StrAddr Type: STRING(24) Description: Board address as string |
| Input Tag | Name: DIP Type: WORD Description: DIP settings |
| Input Tag | Name: UplinkUp Type: BOOL Description: Uplink port is up and running |
| Input Tag | Name: UplinkLOS Type: BOOL Description: Loss of signal in uplink port |
| Input Tag | Name: UplinkErrCount Type: INT Description: Error counter for uplink port |
| Input Tag | Name: UplinkCRCErr Type: BOOL Description: CRC error in uplink data |
| Input Tag | Name: UplinkCRCErrCount Type: INT Description: CRC error in uplink data |
| Input Tag | Name: VCXOCtrl Type: LREAL Description: VCXO control voltage |
| Input Tag | Name: Id Type: TimingSlaveEnum Description: Timing slave identifier |
| Input Tag | Name: Generic Type: TimingSlaveGenericStruct Description: Describes the slave payload (generic) |
| Input Tag | Name: Fanout Type: TimingSlaveFanoutStruct Description: Describes a fanout connected to the port |
| Input Tag | Name: DuoTone Type: TimingSlaveDuoToneStruct Description: Describes a DuoTone connected to the port |
| Input Tag | Name: CFC Type: TimingSlaveCfcStruct Description: Describes a comparator/frequency counter connected to the port |
| Input Tag | Name: XOLock Type: TimingSlaveXoLockingStruct Description: Describes an RF source connected to the port |
| Input Tag | Name: IRIGB Type: TimingSlaveIrigBStruct Description: Describes an IRIG-B connected to the port |

| User Interface Type TYPE TimingSlaveGenericStruct : STRUCT Payload: ARRAY[0..23] OF DWORD; END_STRUCT END_TYPE |  |
| --- | --- |
| Type name | TimingSlaveGenericStruct |
| Description |  |
| Definition | STRUCT |
| Input Tag | Name: Payload Type: ARRAY[0..23] OF DWORD Description: Data from timing slave |

| User Interface Type TYPE TimingSlaveCfcStruct : STRUCT Error: ErrorStruct; HasInput: DWORD; TimeDiff: ARRAY[1..7] OF LREAL; Frequency: ARRAY[1..6] OF LREAL; END_STRUCT END_TYPE |  |
| --- | --- |
| Type name | TimingSlaveCfcStruct |
| Description | This structure describes a comparator/frequency counter slave module. |
| Definition | STRUCT |
| Input Tag | Name: Error Type: ErrorStruct Description: Error handling |
| Input Tag | Name: HasInput Type: DWORD Description: Active inputs (bit encoded) |
| Input Tag | Name: TimeDiff Type: ARRAY[1..7] OF LREAL Description: Comparator difference in sec |
| Input Tag | Name: Frequency Type: ARRAY[1..6] OF LREAL Description: Measured frequency |

| User Interface Type TYPE TimingSlaveDuoToneStruct : STRUCT Error: ErrorStruct; END_STRUCT END_TYPE |  |
| --- | --- |
| Type name | TimingSlaveDuoToneStruct |
| Description | This structure describes a DuoTone slave module. |
| Definition | STRUCT |
| Input Tag | Name: Error Type: ErrorStruct Description: Error handling |

| User Interface Type TYPE TimingSlaveFanoutStruct : STRUCT Error: ErrorStruct; IsMaster: BOOL; HasFanout: BOOL; FanoutPorts: INT; HasOCXO: BOOL; HasExtPPS: BOOL; HasGPS: BOOL; UseExt: BOOL; UseGPS: BOOL; UseUplink: BOOL; GPSLocked: BOOL; OCXOLocked: BOOL; GPSErr: BOOL; GPSErrCount: INT; OCXOCtrl: LREAL; OCXOErr: LREAL; UplinkDelay: LREAL; ExtPPSDelay: LREAL; GPSDelay: LREAL; FanoutUp: WORD; FanoutLOS: WORD; Missing: WORD; DelayErr: WORD; Visual: STRING; END_STRUCT END_TYPE |  |
| --- | --- |
| Type name | TimingSlaveFanoutStruct |
| Description | This structure describes a fanout seen as a slave module through the port. |
| Definition | STRUCT |
| Input Tag | Name: Error Type: ErrorStruct Description: Error handling |
| Input Tag | Name: IsMaster Type: BOOL Description: This is a timing master |
| Input Tag | Name: HasFanout Type: BOOL Description: Fanout ports are available |
| Input Tag | Name: FanoutPorts Type: INT Description: Number of fanout ports |
| Input Tag | Name: HasOCXO Type: BOOL Description: Contains an OCXO |
| Input Tag | Name: HasExtPPS Type: BOOL Description: An external PPS is available |
| Input Tag | Name: HasGPS Type: BOOL Description: Has an internal GPS receiver |
| Input Tag | Name: UseExt Type: BOOL Description: Uses the external PPS for timing source |
| Input Tag | Name: UseGPS Type: BOOL Description: Uses the internal GPS for timing source |
| Input Tag | Name: UseUplink Type: BOOL Description: Uses the uplink for timing source |
| Input Tag | Name: GPSLocked Type: BOOL Description: GPS receiver is locked |
| Input Tag | Name: OCXOLocked Type: BOOL Description: OCXO is locked to timing source |
| Input Tag | Name: GPSErr Type: BOOL Description: GPS error flag |
| Input Tag | Name: GPSErrCount Type: INT Description: Error counter for GPS errors |
| Input Tag | Name: OCXOCtrl Type: LREAL Description: OCXO control voltage |
| Input Tag | Name: OCXOErr Type: LREAL Description: OCXO Error |
| Input Tag | Name: UplinkDelay Type: LREAL Description: Uplink delay in sec |
| Input Tag | Name: ExtPPSDelay Type: LREAL Description: Ext PPS delay in sec |
| Input Tag | Name: GPSDelay Type: LREAL Description: GPS delay in sec |
| Input Tag | Name: FanoutUp Type: WORD Description: Port up and running (bit encoded) |
| Input Tag | Name: FanoutLOS Type: WORD Description: Loss-of-signal in ports (bit encoded) |
| Input Tag | Name: Missing Type: WORD Description: Missing delay in ports (bit encoded) |
| Input Tag | Name: DelayErr Type: WORD Description: Delay error in port (bit encoded) |
| Input Tag | Name: Visual Type: STRING Description: Name of the MFO visual hooked up to the port |

| User Interface Type TYPE TimingSlaveIrigBStruct : STRUCT Error: ErrorStruct; LeapSec: INT; TimeZone: LREAL; DST: BOOL; LeapPend: BOOL; LeapSub: BOOL; IrigErrCountA: INT; IrigDiffA: LREAL; IrigErrCountB: INT; IrigDiffB: LREAL; IrigErrCountC: INT; IrigDiffC: LREAL; END_STRUCT END_TYPE |  |
| --- | --- |
| Type name | TimingSlaveIrigBStruct |
| Description | This structure describes a IRIG-B slave module. |
| Definition | STRUCT |
| Input Tag | Name: Error Type: ErrorStruct Description: Error handling |
| Input Tag | Name: LeapSec Type: INT Description: Number of leap second |
| Input Tag | Name: TimeZone Type: LREAL Description: Local time zone |
| Input Tag | Name: DST Type: BOOL Description: Daylight saving time |
| Input Tag | Name: LeapPend Type: BOOL Description: Leap second is pending |
| Input Tag | Name: LeapSub Type: BOOL Description: Leap second will be subtracted |
| Input Tag | Name: IrigErrCountA Type: INT Description: Error counter in channel A |
| Input Tag | Name: IrigDiffA Type: LREAL Description: IRIG-B time difference in channel A |
| Input Tag | Name: IrigErrCountB Type: INT Description: Error counter in channel B |
| Input Tag | Name: IrigDiffB Type: LREAL Description: IRIG-B time difference in channel B |
| Input Tag | Name: IrigErrCountC Type: INT Description: Error counter in channel C |
| Input Tag | Name: IrigDiffC Type: LREAL Description: IRIG-B time difference in channel C |

| User Interface Type TYPE TimingSlaveXoLockingStruct : STRUCT Error: ErrorStruct; PresetFreq: LREAL; MeasuredFreq: LREAL; HasOCXO: BOOL; OCXOLocked: BOOL; OCXOCtrl: LREAL; OCXOErr: LREAL; END_STRUCT END_TYPE |  |
| --- | --- |
| Type name | TimngSlaveXoLockingStruct |
| Description | This structure describes an RF source slave module. |
| Definition | STRUCT |
| Input Tag | Name: Error Type: ErrorStruct Description: Error handling |
| Input Tag | Name: PresetFreq Type: LREAL Description: Preset frequency |
| Input Tag | Name: MeasuredFreq Type: LREAL Description: Measured frequency |
| Input Tag | Name: HasOCXO Type: BOOL Description: Contains an OCXO |
| Input Tag | Name: OCXOLocked Type: BOOL Description: OCXO is locked to timing source |
| Input Tag | Name: OCXOCtrl Type: LREAL Description: OCXO control voltage |
| Input Tag | Name: OCXOErr Type: LREAL Description: OCXO Error |

| Global constant TimingMagicString: STRING := 'LIGO TIMING SYSTEM VERSION 1.0$R$N'; |  |
| --- | --- |
| Variable name | TimingMagicString |
| Type | STRING |
| Initialization | 'LIGO TIMING SYSTEM VERSION 1.0$R$N' |
| Description | Magic string which marks the beginning of a transmission from the timing master-fanout chassis through the serial port |

| Global constant TimingDataLen: DINT := 2308; |  |
| --- | --- |
| Variable name | TimingDataLen |
| Type | DINT |
| Initialization | 2308 |
| Description | Length of data sent by the master fanout in bytes |

| Global constant TimingFanoutBoardId: DWORD := 16#070011; |  |
| --- | --- |
| Variable name | TimingFanoutBoardId |
| Type | DWORD |
| Initialization | 16#070011 |
| Description | Identification of a master-fanout chassis |

| Global constant TimingSlaveBoardId: DWORD := 16#070071; |  |
| --- | --- |
| Variable name | TimingSlaveBoardId |
| Type | DWORD |
| Initialization | 16#070071 |
| Description | Identification of a slave module |

| Global constant TimingSlaveDuoToneCodeId: DWORD := 16#080335; |  |
| --- | --- |
| Variable name | TimingSlaveDuoToneCodeId |
| Type | DWORD |
| Initialization | 16#080335 |
| Description | Identification of a DuoTone slave |

| Global constant TimingSlaveCfcCodeId: DWORD := 16#070568; |  |
| --- | --- |
| Variable name | TimingSlaveCfcCodeId |
| Type | DWORD |
| Initialization | 16#070568 |
| Description | Identification of a comparator/frequency counter slave |

| Global constant TimingSlaveXoLockingCodeId: DWORD := 16#080665; |  |
| --- | --- |
| Variable name | TimingSlaveXoLockingCodeId |
| Type | DWORD |
| Initialization | 16#080665 |
| Description | Identification of an RF source slave |

| Global constant TimingSlaveIrigBCodeId: DWORD := 16#090305; |  |
| --- | --- |
| Variable name | TimingSlaveIrigBCodeId |
| Type | DWORD |
| Initialization | 16#090305 |
| Description | Identification of an IRIG-B slave |

| Global constant TimingSlaveIrigBCode2Id: DWORD := 16#090030; |  |
| --- | --- |
| Variable name | TimingSlaveIrigBCode2Id |
| Type | DWORD |
| Initialization | 16#090030 |
| Description | Wrong identification of an IRIG-B slave |

| Function Block FUNCTION_BLOCK TimingMasterFanoutFB VAR_INPUT Request: SaveRestoreEnum; END_VAR VAR_IN_OUT ComPortData: TimingSerialPortBufferStruct; MFO: TimingMasterFanoutStruct; MFOInit: TimingMasterFanoutStruct; END_VAR |  |
| --- | --- |
| Name | TimingMasterFanoutFB |
| Description | Main function block to read the serial port information from the serial port and fill out the TimingMasterFanoutStruct. This function block is called from the standard task running at 10ms update rate. |
| Input argument | Name: Request Type: SaveRestoreEnum Description: Request for save/restore/safemode or noop. |
| In/out argument | Name: ComPortData Type: TimingSerialPortBufferStruct Description: Serial port buffers |
| In/out argument | Name: MFO Type: TimingMasterFanoutStruct Description: User interface structure |
| In/out argument | Name: MFOInit Type: TimingMasterFanoutStruct Description: Save/restore variable in persistent memory |

| Function Block FUNCTION_BLOCK TimingSerialPortControlFB VAR_INPUT Mode: ComSerialLineMode_t; pComIn: POINTER TO ARRAY[0..65] OF BYTE; pComOut: POINTER TO ARRAY[0..65] OF BYTE; SizeComIn: UINT; END_VAR VAR_IN_OUT Com: TimingSerialPortBufferStruct; END_VAR |  |
| --- | --- |
| Name | TimingSerialPortControlFB |
| Description | Main function block to communicate with the serial port hadrware. This function block must be called from the fast task running at 1ms update rate. If not buffer overflows will occur. |
| Input argument | Name: Mode Type: ComSerialLineMode_t Description: Should be set to SERIALLINEMODE_EL6_22B for EL6xxx EtherCAT terminals for serial ports |
| Input argument | Name: pComIn Type: POINTER TO ARRAY[0..65] OF BYTE Description: Input buffer of serial port. Should point to a EL6inData22B structure in the input memory region. |
| Input argument | Name: pComOut Type: POINTER TO ARRAY[0..65] OF BYTE Description: Output buffer of serial port. Should point to a EL6outData22B structure in the output memory region. |
| Input argument | Name: SizeComIn Type: UINT Description: Length of input and output buffers |
| In/out argument | Name: Com Type: TimingSerialPortBufferStruct Description: Serial port buffers |

Program Example: (* Global variables *) VAR_GLOBAL CdsTimingComPortDataM5: TimingSerialPortBufferStruct; CdsTimingCOMinM5 AT %I*: EL6inData22B; CdsTimingCOMoutM5 AT %Q*: EL6outData22B; CdsTimingComPortM5: TimingSerialPortControlFB; CdsTimingCMA_AFB: TimingMasterFanoutFB; END_VAR (* Call from fast task with 1 ms update rate *) PROGRAM TimingFast CdsTimingComPortM5 ( Mode := SERIALLINEMODE_EL6_22B, pComIn := ADR (CdsTimingCOMinM5), pComOut := ADR (CdsTimingCOMoutM5), SizeComIn := SIZEOF (CdsTimingCOMinM5), Com := CdsTimingComPortDataM5); END_PROGRAM (* Call from standard task with 10 ms update rate *) PROGRAM Timing VAR SaveRestore: SaveRestoreFB; GotoSafe: BOOL; Request: SaveRestoreEnum; END_VAR SaveRestore( SaveInterval := T#1m, GotoSafe := GotoSafe, Request => Request ); CdsTimingCMA_AFB (ComPortData := CdsTimingComPortDataM5, MFO := H1.Cds.Timing.C.MA_A, Request := Request, MFOInit := CdsTimingCMA_AInit); END_PROGRAM

| Visuals (master-fanout chassis) Master-fanout chassis with GPS receiver (timing master): Master-fanout chassis without GPS receiver (timing fanout): |  |
| --- | --- |
| Name | TimingMasterFanoutVis |
| Description | This is the visual for the timing master-fanout chassis. At the bottom each fanout-put port is listed. They can be turned made active or inactive. A link to the visual of the slave module is also provided. The name of the master-fanout chassis can be chosen freely. |
| Placeholder | Name: MFO Type: TimingMasterFanoutStruct Description: Master-fanout information to be displayed. |

| Visuals (slave modules) Comparator/frequency counter: DuoTone: Fanout: IRIG-B: RF source: |  |
| --- | --- |
| Name | TimingSlaveVis |
| Description | This visual contains the information about a slave module. A link to the previous visual (master-fanout chassis) is also provided. The name of the slave module can be chosen freely. The fanout slave interface also provides a link to the visual of the actual master-fanout chassis hooked up to this port. |
| Placeholder | Name: Port Type: TimingFanoutStruct Description: Port and slave information to be displayed. |